Sri Lanka Institute of Information Technology

IT1040 - Fundamentals of Computing

Year 1, Semester 1- 2026

FreshGuard: IoT-Based Smart Fruit Bowl

Proposal Document

Group 13

| IT Number | Name |
| --- | --- |
| IT26102355 | KULATHUNGA M.A.S.S. (Team Leader) |
| IT26102361 | VINOJA H.W.O. |
| IT26102357 | SUBHASHANA T.O. |
| IT26102368 | PERERA M.K.J.C. |
| IT26102317 | SITHUMINI W.T.I. |
| IT26102348 | DAMAYANTHA H.A.E. |

- Background

Normally, fruits are kept in an open bowl at home. This is a simple and easy method, but a normal fruit bowl cannot give any information about temperature, humidity, gas level changes or the weight changes of fruits. Because of this, people may only notice that the fruit is spoiled after it becomes too soft, changes color, or gives off a bad smell.

Food waste is a common problem in many homes. Sometimes people buy fruit, but they cannot eat it before it gets spoiled. Reducing this unnecessary waste of food can help families to save money and reduce the amount of waste from homes.

Our proposed system is called FreshGuard. It uses sensors and Internet of Things, also known as IoT, to monitor the condition around the fruits. The system will give an early warning to the user when it detects unusual changes.

However, FreshGuard will not scientifically confirm whether the fruit is safe to eat or not. It will only monitor the storage condition and warn the user when sensor values stay outside the selected normal range.

2. Problem and Motivation

2.1 Problem Statement

Traditional fruit bowls cannot measure temperature, humidity, gas level changes or the weight of the fruits. Therefore, users may only notice the spoilage after visible signs are appearing.

Because of this, a simple and affordable monitoring system is needed to observe these changes, warn the user and automatically improve the ventilation when it is required.

2.2 Motivation

The main motivation of this project is to reduce unnecessary fruit waste and make it easy for users to check the condition of their fruits.

This project also gives our group a good opportunity to use the basic knowledge we learned about sensors, microcontrollers, programming and IoT communication in one practical project.

The proposed system will be able to:

- Show live sensor readings using a simple web dashboard.

- Use a fan and an automatic cover to improve the airflow when the condition needs attention.

- Use warning lights and a buzzer to inform the user without always checking the web dashboard.

- Save sensor readings, so the user can check them later.

3. Aim and Objectives

3.1 Aim

The main aim of this project is to design and build an IoT-based smart fruit bowl that can monitor fruit storage conditions, give early warnings, save sensor readings and automatically control the ventilation.

3.2 Objectives

- To connect and test the ESP32, DHT22, MQ-135, load cell, HX711 and other needed components.

- To measure temperature, humidity, gas level changes and fruit weight in regular time intervals.

- To process the sensor readings and classify the fruit condition as Fresh, Attention or Possible Spoilage.

- To control a 5V fan and SG90 servo motor according to the detected condition.

- To give local warnings using an RGB LED and active buzzer.

- To save sensor readings and system activities into a MicroSD card.

- To develop a Wi-Fi-based web dashboard to show live readings and the current system status.

- To test the sensors, actuators, data storage and web dashboard communication

3.3 Scope

FreshGuard will be developed as a small household prototype. It will monitor the environment around the fruits, provide warning messages and control the ventilation.

The system will not identify specific fruit diseases. It also will not provide medical or food safety decisions. It is only designed as an early warning and monitoring system.

4. System Diagram

INPUTS

DHT22
Temperature & Humidity

MQ-135
Gas / VOC Trend

Load Cell + HX711
Weight Change

OUTPUTS

5V Fan
Ventilation

SG90 Servo
Automatic Cover

RGB LED + Buzzer
Local Warning

MicroSD Card
Store Readings

Web Dashboard
Live Monitoring

PROCESS

ESP32 DevKit V1

Read Sensors
Compare Thresholds
Decide System Status
Control Outputs
Send Data by Wi-Fi

The FreshGuard system mainly has three sections. They are input, processing and output.

The input section collects information about temperature, humidity, gas level changes and the weight of the fruits. The ESP32 receives these readings and compares them with the selected threshold values.

According to the results, the output section will control the fan, servo motor, RGB LED and buzzer. The system will also save sensor readings into the MicroSD card and update the web dashboard.

The MQ-135 sensor will only be used to identify changes in the gas level around the fruits. It will not give exact laboratory measurements and it cannot confirm that the fruit is spoiled.

5. Methodology

The FreshGuard prototype will be developed by using simple hardware and software components. The ESP32 will be used as the main controller because it can process the sensor readings and also connect with Wi-Fi.

5.1 Hardware Components

| Component | Purpose |
| --- | --- |
| ESP32 DevKit V1 | Main controller and Wi-Fi connection |
| DHT22 | Measures temperature and humidity |
| MQ-135 | Observes changes in gas/VOC levels |
| Load Cell + HX711 | Measures fruit weight; the HX711 converts the load-cell signal to digital data |
| 5V DC Fan | Provides ventilation |
| SG90 Servo Motor | Opens and closes the automatic cover |
| MOSFET Module | Switches the fan safely from the ESP32 control signal |
| RGB LED and Active Buzzer | Provides local status and warning |
| MicroSD Module | Stores readings and system events |
| 5V Power Supply, Breadboard and Wires | Provides power and prototype connections |

5.2 Software and Technologies

The following software and technologies will be used for this project:

- Arduino IDE for writing and uploading the program.

- Arduino C/C++ for programming the ESP32 using the Arduino IDE.

- HTML, CSS and basic JavaScript for creating the web dashboard.

- Wi-Fi communication for viewing live sensor data.

5.3 Development Process

1. Connect each sensor to the ESP32 and test them separately.

2. Calibrate the DHT22, MQ-135 and load cell by taking repeated readings.

3. Read the temperature, humidity, gas trend and weight values at fixed time intervals.

4. Remove invalid readings and calculate a simple average to reduce sudden sensor changes.

5. Compare the processed readings with the selected threshold values.

6. Set the system status as Fresh, Attention or Possible Spoilage.

7. Turn on the fan, move the servo-controlled cover and activate the warnings when needed.

8. Save the sensor readings and system activities into the MicroSD card.

9. Update the web dashboard with the latest readings and system status.

10. Test the complete prototype and change the threshold values according to the test results.

5.4 Basic Operating Cycle

The system's operating cycle is:   Sense → Process → Decide → Act → Store → Display.

This cycle will repeat while the system is powered on.

First, the sensors will collect data from the environment. Then, the ESP32 will process the data and make a decision. After that, it will control the outputs, save the data and display it through the web dashboard.

5.5 Safety Considerations

The following safety methods will be included in the prototype:

- Electronic components will be placed under the fruit bowl and protected from water and moisture.

- The fan will be covered using a small safety guard.

- The fan will be controlled through a MOSFET module instead of connecting it directly to an ESP32 pin.

- Only low-voltage DC power will be used for the prototype.

- The wires will be properly connected to reduce loose connections and short circuits.

START

Methodology Flowchart

YES

NO

Turn Fan ON ,Open Cover ,Show Warning

Keep Normal Mode

Show Fresh Status

Repeat Monitoring

Wait for Next Reading

Save Data to MicroSD and Update Web Dashboard

Compare Readings with Thresholds

Filter and Process Sensor Readings

Read Temperature, Humidity, Gas and Weight

Initialize ESP32 and Sensors

6. Evaluation Method

After completing the FreshGuard prototype, it will be evaluated using several practical tests.

- Temperature and Humidity Test

The temperature and humidity values from the DHT22 sensor will be compared with a reference meter. This test will be repeated several times to check the accuracy and consistency of the readings.

- Weight Test

Objects with known weights will be placed on the load cell. The readings will be checked to see whether the load cell is correctly calibrated and gives similar values for repeated tests.

- Gas-Trend Test

First, a normal clean-air reading will be recorded as a baseline. After that, the MQ-135 sensor will be placed near ripening fruits to check whether it can identify a clear gas-level change.

This test will only focus on the changes of gas levels. It will not measure the exact amount or type of gas.

- Actuator Test

The fan and servo motor will be tested to confirm that they are working correctly when sensor values reach the selected threshold levels.

The servo motor should open or close the cover, and the fan should turn on when better ventilation is required.

- Alert Test

The RGB LED and buzzer will be checked to make sure they provide the correct warning for each system condition.

For example, different colours can be used for Fresh, Attention and Possible Spoilage conditions.

- Data-Logging Test

The data stored in the MicroSD card will be checked to confirm that sensor readings and system activities are saved correctly.

The group will also check whether the data can be opened and read without any errors.

- Dashboard Test

The web dashboard will be tested to confirm that the latest sensor readings and current system status are updating correctly through Wi-Fi.

The dashboard will also be checked using a phone or computer to make sure the details are easy to understand.

- Reliability Test

The complete system will be operated continuously for a selected time period. During this test, our group will check for system resets, missing readings, sensor errors and Wi-Fi communication failures.

- Usability Test

Group members will use the web dashboard and give their feedback. They will check whether the sensor values, status indicators and warning messages are easy to understand.

Expected Final Result

At the end of the project, the completed FreshGuard prototype is expected to monitor the environment around the fruits and show a simple system status.

It should be able to operate the fan and automatic cover when needed, provide local warnings using the RGB LED and buzzer, save the sensor readings and show the latest information using a web dashboard.

The final prototype may not completely prevent fruits from getting spoiled, but it can help the user to notice unusual changes earlier and manage their fruits in a better way.

7. References

- United Nations Environment Program, "Food Waste Index Report 2024." Available: https://www.unep.org/resources/publication/food-waste-index-report-2024

- Espressif Systems, "ESP32 Series Datasheet." Available: https://documentation.espressif.com/esp32_datasheet_en.pdf

- Adafruit Industries, "DHT11, DHT22 and AM2302 Sensors." Available: https://learn.adafruit.com/dht/overview

- Zhengzhou Winsen Electronics Technology Co., Ltd., "MQ135 Air Quality Gas Sensor Manual." Available: https://www.winsen-sensor.com/d/files/semiconductor/mq135.pdf

- Avia Semiconductor, "HX711: 24-Bit Analog-to-Digital Converter for Weigh Scales." Available: https://cdn.sparkfun.com/datasheets/Sensors/ForceFlex/hx711_english.pdf

- Arduino, "Arduino IDE Documentation." Available: https://docs.arduino.cc/software/ide/

- TRONIC.LK, "Electronic Components and Module Prices." Available:
  https://tronic.lk/shop/products